SECTION 16075 - ELECTRICAL IDENTIFICATION

PART I GENERAL

1.1 SECTION INCLUDES

- Nameplates and labels.

1.2 REFERENCES

- NFPA 70 - National Electrical Code, National Fire Protection Association (edition adopted by Authority

Having Jurisdiction).

1.3 QUALITY ASSURANCE

- Conform to requirements of NFPA 70.

- Products: Listed and classified by Underwriters Laboratories Inc. as suitable for purpose specified and

shown.

PART 2 PRODUCTS

2.1 NAMEPLATES AND LABELS

- Nameplates: Engraved three-layer laminated plastic, while letters on black background.

- Locations:

  - Each electrical distribution and control equipment enclosure.

- Letter Size:

  - Use 1/8 inch (3 mm) letters for identifying individual equipment and loads.

  - Use 1/4 inch (6 mm) letters for identifying grouped equipment and loads.

- Labels (for Power Outlets): Embossed adhesive tape, with 1/8 inch (3 mm) black letters on clear

background. Use only for identification of appliances and equipment with their own branch circuits

and for dedicated computer circuits. Label outlet with name of load, panel and circuit number.

PART 3 EXECUTION

3.1 PREPARATION

- Degrease and clean surfaces to receive nameplates and labels.

3.2 INSTALLATION

- Install nameplates and labels parallel to equipment lines.

- Secure nameplates to equipment front using screws.

- Secure nameplates to inside surface of door on panelboards that are recessed in finished locations.

END OF SECTION