SECTION 09651 - RESILIENT TILE FLOORING

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data and color Samples.

        - Extra Materials:  Deliver to Owner at least 1 box for each 50 boxes or fraction thereof, of each type and color of resilient floor tile installed.

- PRODUCTS

      - VINYL COMPOSITION FLOOR TILE

        - Azrock or approved equal

          - Color and Pattern:  As selected by architect from standard colors.

        - ASTM F 1066, Class 2 (through-pattern tile).

        - Wearing Surface:  Embossed.

        - Thickness:  [1/8 inch (3.2 mm)]

        - Size:  [12 by 12 inches (304.8 by 304.8 mm)]

        - Critical Radiant Flux Classification:  Class I, not less than 0.45 w/sq. cm per ASTM E 648.

      - INSTALLATION ACCESSORIES

        - Trowelable Leveling and Patching Compounds:  Latex-modified, portland cement- or blended hydraulic cement-based formulation provided or approved by flooring manufacturer for applications indicated.

        - Adhesives:  Water-resistant type recommended by manufacturer to suit resilient products and substrate conditions indicated.

        - Metal Edge Strips:  Extruded aluminum with mill finish of width shown, of height required to protect exposed edge of tiles, and in maximum available lengths to minimize running joints.

- EXECUTION

      - INSTALLATION

        - Lay out tiles from center marks established with principal walls, discounting minor offsets, so tile widths at opposite edges of room are equal to one another and are at least one-half of a tile.

        - Match tiles for color and pattern by selecting tiles from cartons in same sequence as manufactured and packaged.  Lay tiles in basket-weave pattern with grain direction alternating in adjacent tiles

END OF SECTION 09651