SECTION 10155 - TOILET COMPARTMENTS

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data, Shop Drawings, and material Samples.

- PRODUCTS

      - TOILET COMPARTMENTS AND SCREENS

        - Bobrick series 1180 or architect approved equal

      - MATERIALS

        - Panel, Pilaster, and Door Material:  As follows:

          - Solid-Plastic, Polymer Resin:  High-density polyethylene (HDPE) with homogenous color throughout.  Provide material not less than 1 inch (25 mm) thick with seamless construction and eased edges. Flame spread rating 20, smoke density 95, ASTM E84

          - Color:  As selected by architect from manufacturers standard colors.

        - Pilaster Shoes and Sleeves (Caps):  Stainless steel or solid plastic, polymer resin, not less than 3 inches (75 mm) high.

        - Brackets:  Continuous.

          - Material: Stainless steel.

      - FABRICATION

        - Toilet Compartments: Floor anchored.

        - Urinal Screens: Floor anchored.

        - Solid-Plastic, Polymer-Resin Units:  Provide aluminum heat-sink strips at exposed bottom edges of panels and doors.

        - Doors:  Unless otherwise indicated, 24-inch- (610-mm-) wide in-swinging doors for standard toilet compartments and 36-inch- (914-mm-) wide out-swinging doors with a minimum 32-inch- (813-mm-) wide clear opening for compartments indicated to be accessible to the disabled.

        - Door Hardware: Stainless steel Provide units that comply with accessibility requirements of authorities having jurisdiction at compartments indicated to be accessible to the disabled.

          - Hinges:  Self-closing type, adjustable to hold door open at any angle up to 90 degrees.

          - Latches and Keepers:  Surface-mounted unit designed for emergency access and with combination rubber-faced door strike and keeper.

          - Coat Hook:  Combination hook and rubber-tipped bumper, sized to prevent door from hitting compartment-mounted accessories.

          - Door Bumper:  Rubber-tipped bumpers at out-swinging doors or entrance screen doors.

          - Door Pull:  Provide at out-swinging doors.  Provide units on both sides of doors at compartments indicated to be accessible to the disabled.

          - All hardware shall be manufacturers Vandal –Resistant construction.

- EXECUTION

      - INSTALLATION

        - Install units rigid, straight, level, and plumb, with not more than 1/2 inch (13 mm) between pilasters and panels and not more than 1 inch (25 mm) between panels and walls.  Provide brackets, pilaster shoes, bracing, and other components required for a complete installation.  Use theft-resistant exposed fasteners finished to match hardware.  Use sex-type bolts for through-bolt applications.

          - Stirrup Brackets:  Align brackets at pilasters with brackets at walls.  Locate wall brackets so holes for wall anchors occur in masonry or tile joints.

          - Set hinges on in-swinging doors to hold open approximately 30 degrees from closed position when unlatched.  Set hinges on out-swinging doors and swing doors in entrance screens to return to fully closed position.

END OF SECTION 10155