SECTION 09512 - ACOUSTICAL TILE CEILINGS

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals: Product Data and material Samples.

        - Acoustical Tile Ceilings: ASTM E 1264 for Class A materials, tested per ASTM E 84.

        - Fire-Rating Ceiling Assemblies: ASTM E 119 and listed in UL's "Fire Resistance Directory," in ITS's "Certification Listings," or in the listing of another qualified testing and inspecting agency.

- PRODUCTS

      - ACOUSTICAL TILE

        - “Cirrus” Armstrong Industries Inc., “Eclipse” USG Interiors inc., “Casshmere” Celotex Corp.

        - Classification: Type III, Form 1.

        - Pattern: Pattern E.

        - Color: As selected.

        - Light Reflectance (LR) Coefficient: .84

        - Noise Reduction Coefficient (NRC): 0.70

        - Ceiling Attenuation Class (CAC): 35

        - Edge Detail: Beveled, kerfed and rabbeted joints.

        - Thickness: 15/16”.

        - Size: 24 by 24 inches (610 by 610 mm).

      - SUSPENSION SYSTEM

        - Ceiling Suspension System: Direct hung; ASTM C 635, intermediate-duty structural classification. By one of the following manufacturers.

          - Armstrong World Industries Inc., USG Interiors Inc., Chicago Metallic Inc.

        - Attachment Devices: Size for 5 times the design load indicated in ASTM C 635, Table 1, Direct Hung, unless otherwise indicated.

        - Wire Hangers, Braces, and Ties: Zinc-coated carbon-steel; ASTM A 641 (ASTM A 641M), Class 1 zinc coating, soft temper.

          - Wire Diameter: Size so its stress at 3 times the hanger design load (ASTM C 635, Table 1, Direct Hung) will be less than the yield stress of wire, but provide not less than 0.106-inch- (2.69-mm-) diameter wire.

        - Access: Identify upward access tile with manufacturer's standard unobtrusive markers for each access unit.

- EXECUTION

      - INSTALLATION

        - Ceiling Suspension System Installation: ASTM C 636.

        - Ceiling Suspension System Installation Requiring Seismic Restraint:

          - UBC Standard 25-2.

          - CISCA's "Recommendations for Direct-Hung Acoustical Tile and Lay-in Panel Ceilings--Seismic Zones 0-2."

          - CISCA's "Guidelines for Seismic Restraint of Direct-Hung Suspended Ceiling Assemblies--Seismic Zones 3 & 4."

          - ASTM E 580.

END OF SECTION 09512