SECTION 02230 - SITE CLEARING

- GENERAL

      - SECTION REQUIREMENTS

        - Traffic:  Minimize interference with adjoining roads, streets, walks, and other adjacent occupied or used facilities during site-clearing operations.

        - Salvable Improvements:  Carefully remove items indicated to be salvaged and store on Owner's premises where indicated.

        - Notify utility locator service for area where Project is located before site clearing.

- PRODUCTS (Not Applicable)

- EXECUTION

      - PREPARATION

        - Protect and maintain benchmarks and survey control points from disturbance.

        - Install erosion and sedimentation control measures before site clearing.

        - Protect site improvements to remain from damage.  Restore damaged improvements to condition existing before start of site clearing.

        - Locate and clearly flag trees and vegetation to remain or to be relocated.

        - Protect remaining trees and shrubs from damage and maintain vegetation.  Employ a licensed arborist to repair tree and shrub damage.  Restore damaged vegetation.  Replace damaged trees that cannot be restored to full growth, as determined by arborist.

        - Do not store materials or equipment or permit excavation within drip line of remaining trees.

        - Locate, identify, disconnect, and seal or cap off utilities indicated to be removed.

      - SITE CLEARING

        - Strip topsoil.  Stockpile topsoil that will be reused in the Work.

        - Remove obstructions, trees, shrubs, grass, and other vegetation to permit installation of new construction.  Removal includes digging out stumps and obstructions and grubbing roots.

        - Remove existing above- and below-grade improvements as indicated and as necessary to facilitate new construction.

        - Remove slabs, paving, curbs, gutters, and aggregate base as indicated.

        - In areas not to be further excavated, fill depressions resulting from site clearing.  Place and compact satisfactory soil materials in 6-inch (150-mm-) thick layers to density of surrounding original ground.

        - Dispose of waste materials, including trash, debris, and excess topsoil, off Owner's property.  Burning waste materials on-site is not permitted.

END OF SECTION 02230