SECTION 09910 - PAINTING

- GENERAL

      - SECTION REQUIREMENTS

        - Summary: Paint all exposed surfaces unless otherwise indicated.

          - Paint the back side of access panels.

          - Color code mechanical piping in accessible ceiling spaces.

          - Do not paint prefinished items, finished metal surfaces, operating parts, labels, and materials obviously intended to be left exposed such as brick and tile.

          - Unless otherwise indicated do not paint concealed surfaces.

        - Submittals: Product Data and color Samples.

        - Mockups: Full-coat finish sample (benchmark sample) of each type of coating, substrate, color, and finish required in area of not less than 100 sq. ft (9 sq. m).  Comply with PDCA P5.

        - Obtain block fillers, primers, and undercoat materials for each coating system from the same manufacturer as the finish coats.

        - Extra Materials: Deliver to Owner a 1-gal. (3.8-L) container, properly labeled and sealed, of each color and type of finish coat paint used on Project.

- PRODUCTS

      - PAINT

        - Sherwin-Williams or Architect approved equal.

          - Colors: As selected by Architect.

        - Material Quality: Manufacturer's best-quality of coating types specified.

        - Material Compatibility: Complete system of compatible components that is recommended by manufacturer for application indicated.

- EXECUTION

      - APPLICATION

        - Comply with paint manufacturer's written instructions for surface preparation, environmental and substrate conditions, product mixing, and application.

      - EXTERIOR PAINT APPLICATION SCHEDULE

        - Concrete, Stucco, and Masonry: As follows:

          - Flat Acrylic: Two coats over primer.

          - Semigloss, Acrylic Enamel:  wo coats over primer.

        - Concrete Masonry Units: As follows:

          - Flat Acrylic: Two coats over block filler.

          - Semigloss, Acrylic Enamel: Two coats over block filler and primer.

        - Exterior Gypsum Soffit Board:  As follows:

          - Flat Acrylic: Two coats over primer.

          - Semigloss, Acrylic Enamel: Two coats over primer.

        - Smooth Wood:  As follows:

          - Low-Luster Acrylic: Two coats over primer.

          - Semigloss, Acrylic Enamel: Two coats over primer.

          - Full-Gloss, Alkyd Enamel: Two coats over primer.

        - Wood Trim:  As follows:

          - Medium-Shade, Semigloss, Acrylic Enamel: Two coats over primer.

          - Medium-Shade, Full-Gloss, Alkyd Enamel: Two coats over primer.

          - Deep-Color, Full-Gloss, Alkyd Enamel: Two coats over primer.

          - Deep-Color, Full-Gloss, Acrylic Enamel: Two coats over primer.

        - Wood Shakes and Rough Siding:  As follows:

          - Flat Acrylic: Two coats acrylic-latex stain.

          - Flat Alkyd: Two coats alkyd/oil stain.

        - Plywood:  As follows:

          - Low-Luster Acrylic: Two coats over primer.

        - Stained Wood:  As follows:

          - Flat Acrylic: One coat semitransparent, waterborne, penetrating wood stain.

          - Flat Alkyd: One coat semitransparent, alkyd, penetrating wood stain.

        - Ferrous Metal:  As follows:

          - Flat, Alkyd Enamel: Two coats over rust-inhibitive primer.

          - Low-Luster Acrylic: Two coats over rust-inhibitive primer.

          - Semigloss, Acrylic Enamel: Two coats over rust-inhibitive primer.

          - Full-Gloss, Alkyd Enamel: Two coats over rust-inhibitive primer.

          - Deep-Color, Full-Gloss, Alkyd Enamel: Two coats over rust-inhibitive primer.

        - Zinc-Coated Metal:  As follows:

          - Low-Luster Acrylic: Two coats over galvanized metal primer.

          - Semigloss, Acrylic Enamel: Two coats over galvanized metal primer.

          - Full-Gloss, Alkyd Enamel: Two coats over galvanized metal primer.

        - Aluminum:  As follows:

          - Semigloss, Acrylic Enamel: Two coats over primer.

          - Full-Gloss, Alkyd Enamel: Two coats over primer.

      - INTERIOR PAINT APPLICATION SCHEDULE

        - Concrete and Masonry: As follows:

          - Flat Acrylic: Two coats over primer.

          - Odorless, Flat Alkyd: Two coats over primer.

          - Low-Luster, Acrylic Enamel: Two coats over primer.

          - Semigloss, Alkyd Enamel: Two coats over primer.

        - Concrete Masonry Units: As follows:

          - Flat Acrylic: Two coats over block filler.

          - Low-Luster, Acrylic Enamel: Two coats over block filler.

          - Semigloss, Alkyd Enamel: Two coats over undercoat and filled surface.

        - Mineral-Fiber-Reinforced Cement Panels: As follows:

          - Flat Acrylic: Two coats.

        - Gypsum Board: As follows:

          - Flat Acrylic: Two coats over primer.

          - Low-Luster, Acrylic Enamel: Two coats over primer.

          - Semigloss, Alkyd Enamel: Two coats over primer.

        - Plaster:  As follows:

          - Flat Acrylic: Two coats over primer.

          - Odorless, Flat Alkyd: Two coats over primer.

          - Low-Luster, Acrylic Enamel: Two coats over primer.

          - Semigloss, Acrylic Enamel: One coat over undercoat and primer.

          - Semigloss, Alkyd Enamel: One coat over undercoat and primer.

          - Full-Gloss, Alkyd Enamel: Two coats over primer.

        - Acoustical Plaster: As follows:

          - Flat, Acrylic Latex: Two coats.

        - Woodwork and Hardboard: As follows:

          - Low-Luster, Acrylic Enamel: Two coats over primer.

          - Semigloss, Acrylic Enamel: Two coats over wood undercoater.

          - Semigloss, Alkyd Enamel: Two coats over primer.

          - Full-Gloss, Alkyd Enamel: Two coats over wood undercoater.

        - Stained Woodwork:  As follows:

          - Alkyd-Based, Satin Varnish: Two coats clear-satin varnish over sealer and wood stain.

          - Waterborne, Satin Varnish: Two coats clear-satin varnish over sealer and wood stain.

          - Water-Based, Full-Gloss Varnish: Two coats full-gloss varnish over sealer and wood stain.

          - Alkyd-Based Stain, Wax-Polished Finish: Three coats paste wax over sealer and wood stain.

        - Natural-Finish Woodwork:  As follows:

          - Alkyd-Based, Satin Varnish: Two coats clear-satin varnish over sealer.

          - Waterborne, Satin Varnish: Two coats clear-satin varnish over sealer.

          - Water-Based, Full-Gloss Varnish: Two coats full-gloss varnish over sealer.

          - Wax-Polished Finish: Three coats paste wax over sealer.

        - Ferrous Metal:  As follows:

          - Flat Acrylic: Two coats over primer.

          - Low-Luster, Acrylic Enamel: Two coats over primer.

          - Semigloss, Acrylic Enamel: One coat over undercoater and primer.

          - Semigloss, Alkyd Enamel: One coat over undercoater and primer.

          - Full-Gloss, Acrylic Enamel: Two coats over primer.

          - Full-Gloss, Alkyd Enamel: Two coats over undercoater and primer.

        - Zinc-Coated Metal:  As follows:

          - Flat Acrylic: Two coats over primer.

          - Semigloss, Acrylic Enamel: Two coats over primer.

          - Semigloss, Alkyd Enamel: One coat over undercoater and primer.

END OF SECTION 09910