SECTION 08211 - FLUSH WOOD DOORS

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals: Samples for factory-finished doors.

        - Quality Standard: NWWDA I.S.1-A.

- PRODUCTS

      - FLUSH WOOD DOORS

        - Doors for Transparent Finish: Custom grade.

          - Faces: White Birch, plain sliced.

          - Veneer Matching: Book and balance match.

          - Pair matching.

          - Continuous matching for doors with transoms.

        - Doors for Opaque Finish: Custom grade.

          - Faces: Any closed-grain hardwood.

        - Interior Veneer-Faced Solid Core Doors: Five-ply, glued-block core.

        - Blocking for Doors with Particleboard Cores: Top rail blocking at closers, bottom rail blocking at kick plates.

        - Fire-Rated Solid Core Doors: Core construction to provide fire-rating indicated, faces and grade to match non-fire-rated doors.

          - Composite blocking where required to eliminate through-bolting hardware.

          - Laminated-edge construction.

      - FABRICATION AND FINISHING

        - Factory fit doors to suit frame-opening sizes indicated and to comply with referenced quality standard.

          - Comply with NFPA 80 for fire-resistance-rated doors.

        - Factory machine doors for hardware that is not surface applied.

        - Cut and trim openings to comply with referenced standards.

          - Trim light openings with moldings indicated.

          - Factory install louvers in prepared openings.

        - Factory finish wood doors with NWWDA Standard grade, System 7, water-reducible acrylic lacquer finish.

- EXECUTION

      - INSTALLATION

        - Install doors to comply with referenced quality standard.

          - Install fire-rated doors to comply with NFPA 80.

        - Align and fit doors in frames with uniform clearances and bevels.  Machine doors for hardware.  Seal cut surfaces after fitting and machining.

        - Align factory-fitted doors in frames for uniform clearances.

        - Repair, refinish, or replace factory-finished doors damaged during installation, as directed by Architect.

END OF SECTION 08211