SECTION 10801 - TOILET AND BATH ACCESSORIES

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data.

- PRODUCTS

      - MATERIALS

        - Stainless Steel:  ASTM A 666, Type 304, No. 4 finish (satin), 0.0312-inch (0.8-mm) minimum nominal thickness, unless otherwise indicated.

        - Chromium Plating:  ASTM B 456, Service Condition Number SC 2 (moderate service).

        - Mirror Glass:  ASTM C 1036, Type I, Class 1, Quality q2, nominal 6.0 mm thick, with silvering, electroplated copper coating, and protective organic coating complying with FS DD-M-411.

        - Fasteners:  Screws, bolts, and other devices of same material as accessory unit, tamper and theft resistant when exposed, and of galvanized steel when concealed.

        - Keys:  Provide universal keys for internal access to accessories for servicing and resupplying.  Provide minimum of six keys to Owner's representative.

      - TOILET AND BATH ACCESSORIES

        - Paper Towel Dispenser as follows:

          - Bobrick B-262.

          - Mounting: Surface.

          - 420 C-fold or 560 multifold towels.

          - Material:  Stainless steel.

          - Lockset:  Tumbler type.

        - Toilet Tissue Dispenser as follows:

          - Bobrick B-6977.

          - Type:  Recessed multi-roll with tumbler lockset.

          - Mounting:  Semi Recessed.

          - Material:  Stainless steel.

          - Operation:  Noncontrol delivery with standard spindle.

          - Capacity:  Designed for 4-1/2- or 5-inch- (114- or 127-mm-) diameter-core tissue rolls.

        - Liquid-Soap Dispenser as follows:

          - Bobrick B-2111.

          - Mounting:  Surface.

          - Capacity:  34 fluid ounces.

          - Materials:  Stainless steel.

          - Stainless-Steel Soap Valve: Designed for dispensing soap in liquid form.

          - Lockset:  Tumbler type.

          - Refill Indicator:  Window type, unbreakable.

        - Grab Bar as follows:

          - Bobrick B-5806 x 36.

          - Material:  Stainless steel, 0.05 inch (1.3 mm) thick.

          - Mounting:  Concealed.

          - Gripping Surfaces: Slip-resistant texture.

          - Outside Diameter:  1-1/4 inches (32 mm) for medium-duty applications.

        - E. Grab Bar as follows:

          - Bobrick B-5806 x 42.

          - Material:  Stainless steel, 0.05 inch (1.3 mm) thick.

          - Mounting:  Concealed.

          - Gripping Surfaces: Slip-resistant texture.

          - Outside Diameter:  1-1/4 inches (32 mm) for medium-duty applications.

        - Mirror Unit as follows:

          - Bobrick B-293.

          - Frame:  Stainless-steel angle, 0.05 inch (1.3 mm) thick].

- EXECUTION

      - INSTALLATION

        - Install accessories using fasteners appropriate to substrate indicated and recommended by unit manufacturer.  Install units level, plumb, and firmly anchored in locations and at heights indicated.

          - Install grab bars to withstand a downward load of at least 250 lbf (1112 N), when tested according to method in ASTM F 446.

        - Adjust accessories for unencumbered, smooth operation and verify that mechanisms function properly.  Replace damaged or defective items.  Remove temporary labels and protective coatings.

END OF SECTION 10801