SECTION 06402 - INTERIOR ARCHITECTURAL WOODWORK

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals: Shop Drawings and Samples showing the full range of colors, textures, and patterns available for each type of finish.

        - Quality Standard:  Architectural Woodwork Institute's "Architectural Woodwork Quality Standards."

        - Environmental Limitations:  Do not deliver or install woodwork until building is enclosed, wet-work is completed, and HVAC system is operating.

- PRODUCTS

      - MATERIALS

        - Hardboard:  AHA A135.4.

        - Medium-Density Fiberboard:  ANSI A208.2.

        - Particleboard:  ANSI A208.1, Grade M-2.

        - Softwood Plywood:  PS 1.

        - Hardwood Plywood and Face Veneers:  HPVA HP-1.

        - High-Pressure Decorative Laminate:  NEMA LD 3.

        - Thermoset Decorative Overlay:  Comply with ALA 1992.

      - CABINET HARDWARE AND ACCESSORY MATERIALS

        - Hardware Standard:  Comply with BHMA A156.9 for items indicated by reference to BHMA numbers or referenced to this standard.

        - Exposed Hardware Finishes:  For exposed hardware, provide finish that complies with BHMA A156.18 for BHMA code number indicated.

          - Finish:  Satin Chrome:  BHMA 626 or BHMA-652.

        - Furring, Blocking, Shims, and Hanging Strips: Softwood or hardwood lumber, kiln dried to 15 percent moisture content.

      - INTERIOR WOODWORK

        - Complete fabrication before shipping to Project site to maximum extent possible.  Disassemble only as needed for shipping and installing.  Where necessary for fitting at Project site, provide for scribing and trimming.

        - Backout or groove backs of flat trim members, kerf backs of other wide, flat members, except for members with ends exposed in finished Work.

        - Install glass to comply with FGMA's "Glazing Manual."  For glass in wood frames, secure glass with removable stops.

        - Wood Cabinets (Casework) for Transparent Finish:  Custom grade.

          - AWI Type of Cabinet Construction:  Flush overlay

          - Wood Species for Exposed Surfaces: sawn/sliced].

          - Drawer Sides and Backs:  Solid hardwood, same species as exposed surfaces

          - Drawer Bottoms:  Hardwood plywood, same species as exposed surfaces.

        - Laminate-Clad Cabinets (Plastic-Covered Casework):  Custom grade.

          - AWI Type of Cabinet Construction:  Flush overlay

          - Laminate Cladding:  Horizontal surfaces other than tops, HGS; postformed surfaces, HGP; vertical surfaces, HGS; Edges, HGS semiexposed surfaces, VGS.

          - Drawer Sides and Backs:  Solid hardwood.

          - Drawer Bottoms:  Hardwood plywood.

        - Plastic-Laminate Countertops:  Custom grade.

          - Laminate Grade:  HGS.

          - Grain Direction:  Parallel to cabinet fronts.

          - Edge Treatment:  \Same as laminate cladding on horizontal surfaces.

      - SHOP FINISHING OF INTERIOR ARCHITECTURAL WOODWORK

        - Finishes:  Same grades as items to be finished.

        - Finish architectural woodwork at the fabrication shop; defer only final touch up until after installation.

          - Apply one coat of sealer or primer to concealed surfaces of woodwork.  Apply two coats to back of paneling.

          - Apply a vinyl wash coat to woodwork made from closed-grain wood before staining and finishing.

          - After staining, if any, apply paste wood filler to open-grain woods and wipe off excess.  Tint filler to match stained wood.

        - Tr

- EXECUTION

      - INSTALLATION

        - Condition woodwork to prevailing conditions before installing.

        - Install woodwork to comply with AWI Section 1700 for grade specified.

        - Install woodwork level, plumb, true, and straight.  Shim as required with concealed shims.  Install to a tolerance of 1/8 inch in 96 inches (3 mm in 2400 mm) for level and plumb.

        - Scribe and cut woodwork to fit adjoining work, seal cut surfaces, and repair damaged finish at cuts.

        - Install trim with minimum number of joints possible, using full-length pieces to the greatest extent possible.  Stagger joints in adjacent and related members.

        - Anchor countertops securely to base units.  Seal space between backsplash and wall.

        - Anchor paneling to supports with concealed panel-hanger clips and by blind nailing on back-up strips, splined-connection strips, and similar associated trim and framing.

        - Stairwork and Rails:  Cut carriages to accurately fit treads and risers and securely anchor to supporting substrates.  Glue treads to risers, and glue and nail treads and risers to carriages.  Glue and wedge treads and risers to housed stringers.  Glue and dowel or pin balusters to treads and railings, and railings to newel posts.

      - CABINET HARDWARE AND ACCESSORY SCHEDULE

        - Butt Hinges:  2-3/4-inch (70-mm), 5-knuckle steel hinges made from 0.095-inch- (2.4-mm-) thick metal; B01361 for flush doors and B01521 for overlay doors.

        - Concealed (European-Type) Hinges:  B01602.

        - Pulls:  Wire pulls, 4 inches (100 mm) long, 5/16 inches (8 mm) in diameter.

        - Catches:  Magnetic catches, B03141

        - Adjustable Shelf Standards:  B04071; with shelf rests, B04081.

        - Drawer Slides:  Side-mounted, zinc-plated steel drawer slides with steel ball bearings, complying with BHMA A156.9, Grade 1 and rated for the following loads:

          - Box Drawer Slides:  75 lbf (330 N).

          - File Drawer Slides:  150 lbf (670 N)

          - Pencil Drawer Slides:  45 lbf (200 N).

        - Door Locks:  E07121.

        - Drawer Locks:  E07041.

        - Grommets for cable passage through countertops:  1-inch (25-mm) OD brown, molded-plastic grommets with brown plastic cap.

        - Paper Slots:  12 inches (305 mm) long by 1-3/4 inches (45 mm) wide by 1 inch (25 mm) deep; brown, molded-plastic, paper-slot liner with 1/4-inch (6-mm) lip.

END OF SECTION 06402