SECTION 09653 - RESILIENT WALL BASE AND ACCESSORIES

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data and color Samples.

        - Extra Materials:  Deliver to Owner at least 10 linear feet (3 linear m) for each 500 linear feet (150 linear m) or fraction thereof, of each type and color of resilient wall base installed.

- PRODUCTS

      - WALL BASE

        - Mercer or approved equal

          - Color and Pattern:  As selected by architect from standard colors

        - Type:  Vinyl, FS SS-W-40, Type II.

        - Style:  Cove with top-set toe.

        - Minimum Thickness: 1/8 inch (3.2 mm).

        - Height: 4 inches (101.6 mm)

        - Lengths:  Coils in lengths standard with manufacturer, but not less than 96 feet (29.26 m).

        - Outside Corners:  Premolded.

        - Inside Corners:  Premolded.

        - Surface:  Smooth.

      - RESILIENT ACCESSORY

        - <.>

          - Color:  As selected by architect from standard colors.

        - Material:  Vinyl.

        - Description:  Carpet edge for glue-down applications, Tile and carpet joiner.

        - Profile and Dimensions:  As indicated

      - INSTALLATION ACCESSORIES

        - Trowelable Leveling and Patching Compounds:  Latex-modified, portland cement- or blended hydraulic cement-based formulation provided or approved by resilient product manufacturer for applications indicated.

        - Stair-Tread-Nose Filler:  Two-part epoxy compound recommended by resilient tread manufacturer to fill nosing substrates that do not conform to tread contours.

        - Adhesives:  Water-resistant type recommended by manufacturer to suit resilient products and substrate conditions indicated.

- EXECUTION

      - INSTALLATION

        - Adhesively install resilient wall base and accessories.

        - Install wall base in maximum lengths possible.  Apply to walls, columns, pilasters, casework, and other permanent fixtures in rooms or areas where base is required.

        - Install reducer strips at edges of flooring that otherwise would be exposed.

END OF SECTION 09653