SECTION 09260 - GYPSUM BOARD ASSEMBLIES

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data.

        - STC-Rated Assemblies:  Provide materials and construction identical to assemblies whose STC ratings were determined according to ASTM E 90 and classified according to ASTM E 413 by a qualified independent testing agency.

        - Fire-Resistance-Rated Assemblies:  Provide materials and construction identical to assemblies tested according to ASTM E 119 by an independent testing and inspecting agency acceptable to authorities having jurisdiction.

- PRODUCTS

      - METAL FRAMING AND SUPPORTS

        - Suspended and Furred Ceilings:  Comply with ASTM C 645 and ASTM C 754.

          - Wire Ties:  ASTM A 641 (ASTM A 641M), Class 1 zinc coating, soft temper, 0.062 inch (1.6 mm) thick.

          - Hangers:  Wire, ASTM A 641 (ASTM A 641M),

          - Carrying Channels:  Cold-rolled steel, [1-1/2 inches (38.1 mm) deep.

          - Furring Channels:  3/4-inch- (19.1-mm-) deep, cold-rolled channels (0.45-mm-) thick in depth channels,

          - Hot-dip galvanized coating complying with ASTM A 653, G40 (ASTM A 653M, Z90) for framing exterior soffits and suspended ceilings within 10 feet (3 m) of exterior walls.

          - Direct-hung grid suspension system for interior ceilings.

        - Partitions:  Comply with ASTM C 645.

          - Studs and Runners:  In depth indicated and 0.0329-inch (0.84-mm) thick, unless otherwise indicated.

          - Rigid Hat-Shaped Furring Channels:  In depth indicated and 0.0179-inch (0.45-mm) thick, unless otherwise indicated.

          - Furring Brackets:  Adjustable serrated-arm type fabricated from corrosion-resistant steel sheet 0.0329-inch (0.84-mm) thick.

          - Resilient Furring Channels:  1/2-inch- (12.7-mm-) deep, with single- or double-leg.

          - Z-Furring:  Z-shaped members with face flange of 1-5/8 inch (31.8 mm), wall-attachment flange of 7/8 inch (22.2 mm), and in depth required by insulation.

          - Hot-dip galvanized coating complying with ASTM A 653, G40 (ASTM A 653M, Z90) for framing members attached to and within 10 feet (3 m) of exterior walls.

      - GYPSUM BOARD

        - Gypsum board products in maximum lengths available to minimize end-to-end butt joints.

          - Gypsum Wallboard:  ASTM C 36, in thickness indicated, with manufacturer's standard edges.  Regular type, unless otherwise indicated, Type X where required for fire-resistance-rated assemblies, Sag-resistant type for ceiling surfaces.

          - Water-Resistant Gypsum Backing Board:  ASTM C 630, in thickness indicated.  Regular type, unless otherwise indicated

          - Glass-Mat, Water-Resistant Gypsum Backing Board:  ASTM C 1178, of thickness indicated. Regular type, unless otherwise indicated,  "Dens-Shield Tile Backer" manufactured by Georgia-Pacific Corp.

      - ACCESSORIES

        - Trim Accessories:  Cornerbead, edge trim, and control joints complying with ASTM C 1047, formed from steel sheet zinc coated by hot-dip process or rolled zinc or plastic.

        - Aluminum Accessories:  Extruded-aluminum accessories indicated with  manufacturer's standard corrosion-resistant primerClass II, color anodic finish; AA-C12C22A32/A34.

        - Gypsum Board Joint Treatment Materials:  Comply with ASTM C 475.  Paper reinforcing tape and setting-type taping compound and drying-type, ready-mixed, compounds for topping.

        - Cementitious Backer Units:  ANSI A118.9.

        - Cementitious Backer Unit Joint Treatment Materials:  Comply with ASTM C 475.

        - Acoustical Sealant for Exposed and Concealed Joints:  Nonsag, paintable, nonstaining latex sealant complying with ASTM C 834.

        - Sound-Attenuation Blankets:  Unfaced mineral-fiber-blanket insulation complying with ASTM C 665 for Type I.

        - Miscellaneous Materials:  Auxiliary materials for gypsum board construction that comply with referenced standards.

- EXECUTION

      - INSTALLATION

        - Install steel framing to comply with ASTM C 754 and with ASTM C 840 requirements that apply to framing installation.

          - Isolate steel framing from building structure, except at floor, to prevent structural movement from transferring loading to partitions.

          - Where studs are installed directly against exterior walls, install asphalt felt strips or foam gaskets between studs and wall.

        - Install and finish gypsum panels to comply with ASTM C 840 and GA-216.

          - Isolate the perimeter of non-load-bearing gypsum board partitions where they abut structural elements, except floors, by providing a 1/4- to 1/2-inch- (6.4- to 12.7-mm-) wide space between gypsum board and the structure.  Trim edges with U-bead edge trim where edges of gypsum panels are exposed.  Seal joints between edges and abutting structural surfaces with acoustical sealant.

          - STC-Rated Assemblies:  Comply with ASTM C 919 for location of edge trim and closing off sound-flanking paths around or through gypsum board assemblies.

          - Fire-Resistance-Rated Assemblies:  Comply with requirements of listed assemblies.

          - Cementitious Backer Units:  Comply with ANSI A108.11.

          - Single-Layer Fastening Methods:  Fasten gypsum panels to supports with screws

        - Finishing Gypsum Board Assemblies:  Level 4 finish, unless otherwise indicated, Level 1 finish for concealed areas, unless a higher level of finish is required for fire-resistance-rated assemblies,  Level 2 finish where panels form substrates for tile.

END OF SECTION 09260