SECTION 08710 - DOOR HARDWARE

- GENERAL

      - SECTION REQUIREMENTS

Delete paragraph below if an allowance is not used.

        - Allowances:  Provide hardware under Hardware Allowance in Division 1 Section "Price and Payment Procedures."

        - Submittals:  Hardware Schedule.

        - Deliver keys to Owner.

        - For fire-rated openings provide hardware tested and listed by UL or FM (NFPA 80).  On exit devices provide UL or FM label indicating "Fire Exit Hardware."

- PRODUCTS

      - HARDWARE

        - Hinges:  Provide the following:

Select from options below.

          - Stainless-steel or brass/bronze hinges with stainless-steel pins for exterior.

          - Nonremovable hinge pins for exterior and public interior exposure.

          - Ball-bearing hinges for doors with closers and entry doors.

          - Two hinges for 1-3/8-inch- (35-mm-) thick wood doors.

          - Three hinges for 1-3/4-inch- (45-mm-) thick doors 90 inches (2300 mm) or less in height; four hinges for doors more than 90 inches (2300 mm) in height.

        - Locksets and latchsets as follows:

          - BHMA A156.2, Series 4000, Grade [1] [2] [3] for bored locks and latches.

          - BHMA A156.3, Grade 1 for exit devices.

          - BHMA A156.5, Grade [1] [2] [3] for auxilliary locks.

          - BHMA A156.12, Series 5000, Grade [1] [2] [3] for interconnected locks and latches.

          - BHMA A156.13, Series 1000, Grade [1] [2] [3] for mortise locks and latches.

          - [Knobs] [Lever handles] on locksets and latchsets, <Insert design>.

          - Provide trim on exit devices matching locksets.

        - Key locks to Owner's [new] [existing] master-key system.

          - Cylinders with seven-pin tumblers and removable cores.

          - Provide cylinders for interior office doors, storefront doors, exterior mechanical room doors, and other locking doors that do not require other hardware.

          - Provide construction keying.

          - Provide key control system, including cabinet.

        - Closers:  Provide the following:

Select from options below.

          - Mount closers on interior side (room side) of door opening.  Provide regular-arm, parallel-arm, or top-jamb-mounted closers as necessary.

          - Adjustable delayed opening (accessible to the disabled) feature on closers.

        - Provide wall stops or floor stops for doors without closers.

        - Provide hardware finishes as follows:

          - Hinges:  [Primed for painting except at exterior doors] [Matching finish of lockset/latchset].

          - Locksets, Latchsets, and Exit Devices:  [Bright brass, clear coated] [Satin brass, clear coated] [Oil-rubbed, oxidized bronze] [Satin chrome plated] [Bright chrome plated]; [at toilet rooms, provide split finish with bright chrome-plated finish on inside].

          - Closers:  [Aluminum enamel] [Bronze enamel] [Primed for field painting] [Matching finish of lockset/latchset].

          - Other Hardware:  Matching finish of lockset/latchset.

- EXECUTION

      - INSTALLATION

        - Mount hardware in locations recommended by the Door and Hardware Institute, unless otherwise indicated.

      - HARDWARE SCHEDULE

Delete this Hardware Schedule if an allowance is used.

Below are examples with various choices for editing to create a hardware schedule.  Copy and edit hardware sets to suit Project.  Function designations are based on BHMA standards.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Interconnected [entry lock (F95)] [entry lock (F96)] [entry lock (F97)] [entry lock (F104)] [entry lock (F105)] [handle set trim (F102)] [handle set trim (F103)] [dormitory lock (F99)] [dormitory lock (F106)] [hotel motel lock (F100)] [hotel motel lock (F101)].

          - Closer.

          - Door knocker.

          - Door viewer.

          - Threshold and weatherstripping.

Hardware sets above and below are for residential entrances.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Bored [entry handleset (F94)] [entry handleset (F107)] [entry handleset (F108)].

          - Bored auxilliary deadlock, [key outside, thumbturn inside] [key both sides].

          - Closer.

          - Door knocker.

          - Door viewer.

          - Threshold and weatherstripping.

Hardware set below is for residential entrances or for nonresidential exterior or interior doors.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Mortise [entry lock (F04)] [front door lock (F08)] [apartment, exit, or public toilet lock (F09)] [apartment corridor door lock (F10)] [dormitory or exit lock (F11)] [dormitory or exit lock (F12)] [dormitory or exit lock (F13)] [store door lock (F14)] [hotel guest lock (F15)] [apartment corridor door lock (F20)].

          - Closer.

          - Door knocker.

          - Door viewer.

          - Threshold and weatherstripping.

Hardware set below is for residential or nonresidential interior door.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Bored [passage or closet latch (F75)] [privacy, bedroom or bath lock (F76)] [entry lock (F81)] [entry lock(F82)] [classroom lock (F84)] [store room or closet lock (F86)].

          - Closer.

Hardware set below is for toilet room doors that do not require locks or latches.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Pull with backing plate and push plate.

          - Closer.

          - [Two] kick plate[s].

Hardware set below is for pair of doors that do not require exit devices.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Mortise [entry lock (F04)] [storeroom or closet lock (F07)] [front door lock (F08)] [dormitory or exit lock (F11)] [dormitory or exit lock (F12)] [dormitory or exit lock (F13)] [store door lock (F14)] [room door lock (F21)].

          - Pair [manual] [automatic] [self-latching] flush bolts with dustproof strike.

          - Two closers.

          - Coordinator.

          - Astragal.

          - [Two] [Four] [kick plates] [armor plates].

          - Threshold and weatherstripping.

Hardware set below is for pair of doors at place of assembly where locking or latching is not required.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Two pulls with backing plates and two push plates.

          - Two closers.

          - [Two] [Four] kick plates.

Hardware set below is for pair of doors where exit devices are required.

        - Hardware Set No. <Insert number>:

          - Hinges.

          - Two surface vertical rod exit devices, with trim, [exit only (01)] [entrance by trim when dogged (02)] [entrance by thumbpiece, key locks or unlocks thumbpiece (05)] [entrance by thumbpiece when unlocked or key and when locked (07)] [entrance by lever when unlocked or key and lever when locked (10)].

          - Two closers.

          - [Two] [Four] kick plates.

        - Hardware Set No. <Insert number>:

          - Bifold door set including overhead track, hangers, pivots, hinges, and knobs.

        - Hardware Set No. <Insert number>:

          - Bipassing sliding door set including overhead track, hangers, floor guide, and flush pulls.

        - Hardware Set No. <Insert number>:

          - Pocket sliding door set including pocket framing, overhead track, hangers, flush pulls, and edge pull.

END OF SECTION 08710