SECTION 09310 - CERAMIC TILE

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data and Samples.

        - Floor Tiles:  Static coefficient of friction not less than 0.6 for level surfaces and 0.8 for ramps, per ASTM C 1028.

- PRODUCTS

      - CERAMIC TILE

        - Ceramic tile that complies with standard grade requirements of ANSI A137.1, "Specifications for Ceramic Tile."

        - Ceramic Mosaic Floor Tile:  Glazed cushion-edged tile, All Shower Floors

          - DalTile Sandalo Series as basis for design

          - Module Size:  1 by 1 inch (25.4 by 25.4 mm)

          - Color:  Universal Mosaic SW 96.

          - Surface Finish:  Abrasive grain surface.

          - Tiles mounted, by manufacturer's standard method, into sheets.

        - Ceramic Mosaic Floor Tile:  Glazed cushion-edged tile, All toilet rm, floors.

          - DalTile Sandalo Series as basis for design

          - Module Size: 6 by 6 inches (152 by 152 mm)]..

          - Color:  Acacia Beige SW91.

          - Surface Finish:  Abrasive grain surface

          - Tiles mounted, by manufacturer's standard method

        - Glazed Wall Tile:  Cushion-edged, flat tile.

          - DalTile Sandalo Series as basis for design

          - Module Size:  6 by 6 inches (152 by 152 mm).

          - Color: Acacia Beige SW91.

          - Surface Finish:  Standard Finish

          - Tiles mounted, by manufacturer's standard method

        - Tile trim units that match characteristics of adjoining flat tile.  Bullnose, Cove base

        - Where indicated, protect exposed surfaces of tile against adherence of mortar and grout by factory precoating them with a hot-applied continuous film of petroleum paraffin wax.  Do not coat unexposed tile surfaces.

      - INSTALLATION MATERIALS

        - Setting and Grouting Materials:  Comply with material standards in ANSI's "Specifications for the Installation of Ceramic Tile" that apply to materials and methods indicated.

          - Bonsal W.R. Company, Mapei Corporation, Tec specialty products inc.

          - Grout Color:  As selected by architect from manufacturers standard colors .

        - Setting-Bed Accessories:  Comply with ANSI A108.1A.

        - Cementitious Backer Units:  Complying with ANSI A118.9, of thickness indicated.

      - STONE THRESHOLDS

        - Granite thresholds complying with ASTM C 615  White, honed marble thresholds complying with the Marble Institute of America's Group A requirements for soundness, and with ASTM C 503 fabricated to be not more than 1/2 inch (12.7 mm) above adjoining finished floor surfaces, with transition edges beveled on a slope of no greater than 1:2.

- EXECUTION

      - INSTALLATION

        - Comply with tile installation standards in ANSI's "Specifications for the Installation of Ceramic Tile" that apply to materials and methods indicated.

        - Comply with TCA's "Handbook for Ceramic Tile Installation."

Select applicable methods in two subparagraphs below; refer to TCA's Handbook for guidance.

          - Floor Tile Installation Method[s]:  TCA F113 thin-set mortar bonded to concrete slab, basis of design Mapei-Ultraflex-1

          - Wall Tile Installation Method[s]: W242 (organic adhesive over gypsum board on metal studs), B414 and B415 and TCA W244 (shower receptor with thin-set mortar bonded to cementitious backer units).

        - At showers, tubs, and where indicated, provide cementitious backer units and treat joints to comply with ANSI A108.11.

        - Lay tile in grid pattern, unless otherwise indicated.  Align joints where adjoining tiles on floor, base, walls, and trim are the same size.

        - Perform cutting and drilling of tile without marring visible surfaces.  Carefully grind cut edges of tile abutting trim, finish, or built-in items for straight aligned joints.  Fit tile closely to electrical outlets, piping, fixtures, and other penetrations so plates, collars, or covers overlap tile.

END OF SECTION 09310